TITLE: -  LPC DECODE IO RANGE

AIM: -

To List LPC decode IO range by Assembly program

ALGORITHM: -

·  Initialize data segment.

·  Display start message.

·  Load PCI address into EAX.

·  Write address to port 0CF8h.

·  Read data from port 0CFCh into EAX.

·  Convert EAX value to hexadecimal string.

·  Display result message.

·  Display hexadecimal value.

PROGRAM:-

TITLE   LPC.ASM -- Read and display LPC Decode IO Range

;//***********************************************************************

;//* *

;//* Copyright (c) 1985-2022, American Megatrends International LLC.     *

;//* *

;//* All rights reserved. Subject to AMI licensing agreement.            *

;//* *

;//***********************************************************************

;<AMI_FHDR_START>

;

; Header:

; Author: Muthuganesh s

; Description: Read LPC GEN1_DEC Register (Bus 0, Dev 31, Func 0, Offset 84h)

; Date: 29/03/2026

;

;<AMI_FHDR_END>

;*************************************************************************

.model small

.386                ; REQUIRED: Allows use of 32-bit registers (EAX) in 16-bit mode

.stack 100h

;----------------------------------------------------------------------------

;       DATA SEGMENT

;----------------------------------------------------------------------------

data segment

msg_start db 13, 10, "Reading LPC GEN1_DEC Register (Bus 0, Dev 31, Fun 0, Off 84h)...$"

msg_res   db 13, 10, 13, 10, "GEN1_DEC Value: 0x$"

;--------------------------------------------------

; Buffer for 32-bit Hexadecimal Output String

;--------------------------------------------------

hex_buf   db 8 dup('0')  ; 8 bytes for hex characters

db '$'         ; DOS string terminator

data ends

;----------------------------------------------------------------------------

;----------------------------------------------------------------------------

;       CODE SEGMENT

;----------------------------------------------------------------------------

code segment

assume cs:code, ds:data

start:

;--------------------------------------------------

; Initialize Data Segment

;--------------------------------------------------

mov ax, data

mov ds, ax

;--------------------------------------------------

; Print Start Message

;--------------------------------------------------

mov dx, offset msg_start

mov ah, 09h

int 21h

;--------------------------------------------------

; 1. Calculate and Write PCI Configuration Address

; Format: Enable (Bit 31) | Bus 0 | Dev 31 | Func 0 | Offset 84h

; Enable = 80000000h

; Dev 31 = 0000F800h

; Offset = 00000084h

; Address = 8000F884h

;--------------------------------------------------

mov dx, 0CF8h           ; Point DX to PCI Config Address Port

mov eax, 8000F884h      ; Load the GEN1_DEC address

out dx, eax             ; Write address to port

;--------------------------------------------------

; 2. Read the PCI Data

;--------------------------------------------------

mov dx, 0CFCh           ; Point DX to PCI Config Data Port

in eax, dx              ; Read 32-bit register value into EAX

;--------------------------------------------------

; 3. Convert EAX (Binary) to Hexadecimal String

;--------------------------------------------------

mov di, offset hex_buf  ; Point DI to our output buffer

mov cx, 8               ; 8 nibbles (hex digits) in a 32-bit register

hex_convert_loop:

rol eax, 4              ; Rotate highest 4 bits (nibble) into AL

push eax                ; Save the full EAX state

and al, 0Fh             ; Mask out everything except the bottom nibble

add al, '0'             ; Add ASCII '0'

cmp al, '9'             ; Is it a number (0-9)?

jle store_hex_char      ; If yes, jump to store it

add al, 7               ; If no, add 7 to convert to 'A'-'F'

store_hex_char:

mov [di], al            ; Write the ASCII character to the buffer

inc di                  ; Move to the next byte in the buffer

pop eax                 ; Restore EAX for the next rotation

loop hex_convert_loop   ; Decrement CX, loop if not zero

;--------------------------------------------------

; 4. Print Result Message and Hex Value

;--------------------------------------------------

mov dx, offset msg_res

mov ah, 09h

int 21h

mov dx, offset hex_buf

mov ah, 09h

int 21h

; Print a couple of newlines for clean exit

mov dl, 13

mov ah, 02h

int 21h

mov dl, 10

mov ah, 02h

int 21h

;--------------------------------------------------

; Exit program

;--------------------------------------------------

exit_program:

mov ah, 4Ch

int 21h

code ends

end start

;----------------------------------------------------------------------------

OUTPUT: -