TITLE: -  CPU INFORMATION

AIM: -

To write a program to find info of the processor like serial number, type, model, stepping.

ALGORITHM: -

·  Initialize data segment.

·  Execute CPUID with EAX=0 to get Vendor ID.

·  Store and display Vendor ID string.

·  Execute CPUID with EAX=1 to get processor signature.

·  Extract Stepping, Model, and Type via bitmasking.

·  Convert extracted values to ASCII hexadecimal characters.

·  Display CPU Type, Model, and Stepping values.

·  Terminate program execution.

PROGRAM:-

TITLE   CPUINFO.ASM -- Read and Display Processor Information

;//***********************************************************************

;//*                                                                     *

;//*   Copyright (c) 1985-2025, American Megatrends International LLC.   *

;//*                                                                     *

;//*      All rights reserved. Subject to AMI licensing agreement.       *

;//*                                                                     *

;//***********************************************************************

;<AMI_FHDR_START>

;

; Header:

; Author: Muthuganesh S

; Description:

; Program to display processor vendor ID, processor type,

; model and stepping using CPUID instruction.

;

;<AMI_FHDR_END>

.model small

.386

.stack 100h

.586

.model small    ; Fixed: Changed from flat to small for 16-bit DOS

.stack 100h

;----------------------------------------------------------------------------

;       DATA     S E G M E N T

;----------------------------------------------------------------------------

data segment

msg1 db 13,10,'Processor Vendor ID : $'

msg2 db 13,10,'Processor Type      : $'

msg3 db 13,10,'Processor Model     : $'

msg4 db 13,10,'Processor Stepping  : $'

vendor db 13 dup('$')

typeval db ?

modelval db ?

stepval db ?

data ends

;----------------------------------------------------------------------------

;----------------------------------------------------------------------------

;       CODE     S E G M E N T

;----------------------------------------------------------------------------

code segment

assume cs:code, ds:data

start:

main proc

mov ax, data

mov ds, ax

call customfunction

mov ax,4C00h

int 21h

main endp

customfunction proc

; --- Get Vendor ID ---

mov eax,0

cpuid

mov dword ptr vendor[0], ebx

mov dword ptr vendor[4], edx

mov dword ptr vendor[8], ecx

mov ah,09h

lea dx,msg1

int 21h

mov ah,09h

lea dx,vendor

int 21h

; --- Get CPU Info ---

mov eax,1

cpuid

; Get Stepping (Bits 0-3)

mov ebx,eax

and bl,0Fh

mov stepval,bl

; Get Model (Bits 4-7)

mov ebx,eax

shr ebx,4       ; Fixed: Shift 32-bit register ebx

and bl,0Fh

mov modelval,bl

; Get Type (Bits 12-13)

mov ebx,eax

shr ebx,12      ; Fixed: Shift 32-bit register ebx

and bl,03h

mov typeval,bl

; --- Print Results ---

; Type

mov ah,09h

lea dx,msg2

int 21h

mov dl,typeval

add dl,30h

mov ah,02h

int 21h

; Model

mov ah,09h

lea dx,msg3

int 21h

mov dl,modelval

add dl,30h      ; Prints symbol if > 9

mov ah,02h

int 21h

; Stepping

mov ah,09h

lea dx,msg4

int 21h

mov dl,stepval

add dl,30h      ; Prints symbol if > 9

mov ah,02h

int 21h

ret

customfunction endp

code ends

end start

OUTPUT: -