TITLE: -  LEGACY IO

AIM: -

To display a text by writing values to port 60h/64h

ALGORITHM: -

·  Initialize the data segment.

·  Display the start message.

·  Read the Keyboard Controller status from port 64h and loop until the input buffer is empty (Bit 1 is 0).

·  Write the command D2h (Write Output Buffer) to the Keyboard Controller command port 64h.

·  Read the status from port 64h again and wait until the input buffer is empty.

·  Write the target data byte ('A') to the Keyboard Controller data port 60h.

·  Display the predefined text string on the screen using DOS interrupts.

·  Display the success message.

·  Terminate program execution.

PROGRAM:-

TITLE   KBCW.ASM -- Display Text by Writing to Port 60h/64h

;//***********************************************************************

;//*                                                                     *

;//*   Copyright (c) 1985-2025, American Megatrends International LLC.   *

;//*                                                                     *

;//*      All rights reserved. Subject to AMI licensing agreement.       *

;//*                                                                     *

;//***********************************************************************

;<AMI_FHDR_START>

;

; Header:

; Author: Muthuganesh S

; Description:

; Program to send commands/data through Keyboard Controller

; ports 60h and 64h and display text on screen.

;

;<AMI_FHDR_END>

.model small

.386

.stack 100h

;----------------------------------------------------------------------------

;       DATA SEGMENT

;----------------------------------------------------------------------------

data segment

msg1 db 13,10,"Writing Data to Keyboard Controller Ports 64h and 60h",13,10,'$'

msg2 db 13,10,"Text Displayed Successfully",13,10,'$'

text db 'HELLO BIOS','$'

data ends

;----------------------------------------------------------------------------

;----------------------------------------------------------------------------

;       CODE SEGMENT

;----------------------------------------------------------------------------

code segment

assume cs:code, ds:data

start:

mov ax,data

mov ds,ax

;--------------------------------------------------

; Display Start Message

;--------------------------------------------------

mov dx,offset msg1

mov ah,09h

int 21h

;--------------------------------------------------

; Wait Until Keyboard Controller Input Buffer Empty

; Status Port = 64h

; Bit1 = Input Buffer Status

;--------------------------------------------------

wait_input_empty:

in al,64h

test al,02h

jnz wait_input_empty

;--------------------------------------------------

; Send Command to Keyboard Controller

; Command D2h:

; Write Output Buffer

;--------------------------------------------------

mov al,0D2h

out 64h,al

;--------------------------------------------------

; Wait Again

;--------------------------------------------------

wait_input_empty2:

in al,64h

test al,02h

jnz wait_input_empty2

;--------------------------------------------------

; Send Data Byte to Port 60h

;--------------------------------------------------

mov al,'A'

out 60h,al

;--------------------------------------------------

; Display Text Normally

;--------------------------------------------------

mov dx,offset text

mov ah,09h

int 21h

;--------------------------------------------------

; Success Message

;--------------------------------------------------

mov dx,offset msg2

mov ah,09h

int 21h

;--------------------------------------------------

; Exit Program

;--------------------------------------------------

mov ah,4Ch

int 21h

code ends

end start

OUTPUT: -

AIM: -

To check and display whether the CMOS checksum is proper or not.

ALGORITHM: -

·  Initialize the data segment.

·  Display the start message.

·  Initialize a checksum accumulator (register BX) to zero.

·  Loop through CMOS memory addresses from 10h to 2Dh.

·  For each address, write the address to CMOS index port 70h, read the data byte from data port 71h, and add it to the accumulator.

·  Save the final accumulated value as the calculated checksum.

·  Read the stored checksum high byte from CMOS address 2Eh and the low byte from address 2Fh (using ports 70h and 71h).

·  Combine both bytes to form the stored 16-bit checksum.

·  Display the stored CMOS checksum in hexadecimal format.

·  Display the calculated CMOS checksum in hexadecimal format.

·  Compare the calculated checksum with the stored checksum.

·  If the checksums match, display the "PROPER" validation message.

·  If the checksums do not match, display the "INVALID" validation message.

·  Terminate program execution.

PROGRAM:-

TITLE   CHK.ASM -- Verify CMOS Checksum

;//***********************************************************************

;//*                                                                     *

;//*   Copyright (c) 1985-2025, American Megatrends International LLC.   *

;//*                                                                     *

;//*      All rights reserved. Subject to AMI licensing agreement.       *

;//*                                                                     *

;//***********************************************************************

;<AMI_FHDR_START>

;

; Header:

; Author: Muthuganesh S

; Description:

; Program to calculate CMOS checksum and verify

; whether CMOS checksum is proper or not.

;

;<AMI_FHDR_END>

.model small

.386

.stack 100h

;----------------------------------------------------------------------------

;       DATA SEGMENT

;----------------------------------------------------------------------------

data segment

msg1 db 13,10,"Checking CMOS Checksum...",13,10,'$'

msg2 db 13,10,"Stored CMOS Checksum     : $"

msg3 db 13,10,"Calculated CMOS Checksum : $"

msg_ok db 13,10,13,10,"CMOS Checksum Status : PROPER",13,10,'$'

msg_bad db 13,10,13,10,"CMOS Checksum Status : INVALID",13,10,'$'

hexbuf db '0000$'

stored_chk dw ?

calc_chk dw ?

data ends

;----------------------------------------------------------------------------

;----------------------------------------------------------------------------

;       CODE SEGMENT

;----------------------------------------------------------------------------

code segment

assume cs:code, ds:data

start:

mov ax,data

mov ds,ax

;--------------------------------------------------

; Display Start Message

;--------------------------------------------------

mov dx,offset msg1

mov ah,09h

int 21h

;--------------------------------------------------

; Calculate CMOS Checksum

;

; Standard AT CMOS checksum area:

; 10h to 2Dh

;

; Stored checksum:

; 2Eh = High Byte

; 2Fh = Low Byte

;--------------------------------------------------

xor bx,bx

mov si,10h

checksum_loop:

mov al,si

out 70h,al

in al,71h

xor ah,ah

add bx,ax

inc si

cmp si,2Eh

jne checksum_loop

mov calc_chk,bx

;--------------------------------------------------

; Read Stored CMOS Checksum

;--------------------------------------------------

mov al,2Eh

out 70h,al

in al,71h

mov ah,al

mov al,2Fh

out 70h,al

in al,71h

mov stored_chk,ax

;--------------------------------------------------

; Display Stored Checksum

;--------------------------------------------------

mov dx,offset msg2

mov ah,09h

int 21h

mov ax,stored_chk

call PrintHexWord

;--------------------------------------------------

; Display Calculated Checksum

;--------------------------------------------------

mov dx,offset msg3

mov ah,09h

int 21h

mov ax,calc_chk

call PrintHexWord

;--------------------------------------------------

; Compare Checksums

;--------------------------------------------------

mov ax,stored_chk

cmp ax,calc_chk

jne checksum_invalid

checksum_valid:

mov dx,offset msg_ok

mov ah,09h

int 21h

jmp exit_program

checksum_invalid:

mov dx,offset msg_bad

mov ah,09h

int 21h

exit_program:

;--------------------------------------------------

; Exit Program

;--------------------------------------------------

mov ah,4Ch

int 21h

;----------------------------------------------------------------------------

; PrintHexWord Procedure

;----------------------------------------------------------------------------

PrintHexWord proc

push ax

push bx

push cx

push dx

push si

mov si,offset hexbuf

mov cx,4

hex_loop:

rol ax,4

mov bl,al

and bl,0Fh

cmp bl,9

jbe digit

add bl,7

digit:

add bl,'0'

mov [si],bl

inc si

loop hex_loop

mov dx,offset hexbuf

mov ah,09h

int 21h

pop si

pop dx

pop cx

pop bx

pop ax

ret

PrintHexWord endp

code ends

end start

OUTPUT: -