TITLE: -  CONVENTION MEMORY

AIM: -

To write a program to find info of the Keyboard special keys state and display the Key state ( On/Off)

ALGORITHM: -

·  Initialize data segment.

·  Display the header message.

·  Execute BIOS INT 16h (Function 02h) to read keyboard status flags.

·  Save the keyboard flags returned in the AL register.

·  Test bit 6 and display Caps Lock status (ON/OFF).

·  Test bit 5 and display Num Lock status (ON/OFF).

·  Test bit 4 and display Scroll Lock status (ON/OFF).

·  Test bit 7 and display Insert Key status (ON/OFF).

·  Test bits 0 and 1 and display Shift Key status (ON/OFF).

·  Test bit 2 and display Ctrl Key status (ON/OFF).

·  Test bit 3 and display Alt Key status (ON/OFF).

·  Terminate program execution.

PROGRAM:-

TITLE   KeySp.ASM -- Display Keyboard Special Key Status

;//***********************************************************************

;//*                                                                     *

;//*   Copyright (c) 1985-2025, American Megatrends International LLC.   *

;//*                                                                     *

;//*      All rights reserved. Subject to AMI licensing agreement.       *

;//*                                                                     *

;//***********************************************************************

;<AMI_FHDR_START>

;

; Header:

; Author: Muthuganesh S

; Description:

; Program to read and display keyboard special key states

; like Caps Lock, Num Lock, Scroll Lock, Insert,

; Shift, Ctrl and Alt using BIOS keyboard status.

;

;<AMI_FHDR_END>

.model small

.386

.stack 100h

;----------------------------------------------------------------------------

;       DATA SEGMENT

;----------------------------------------------------------------------------

data segment

msg1 db 13,10,"Keyboard Special Key Status",13,10,13,10,'$'

capsmsg    db "Caps Lock    : $"

nummsg     db 13,10,"Num Lock     : $"

scrollmsg  db 13,10,"Scroll Lock  : $"

insertmsg  db 13,10,"Insert Key   : $"

shiftmsg   db 13,10,"Shift Key    : $"

ctrlmsg    db 13,10,"Ctrl Key     : $"

altmsg     db 13,10,"Alt Key      : $"

onmsg  db "ON$"

offmsg db "OFF$"

keystate db ?

data ends

;----------------------------------------------------------------------------

;----------------------------------------------------------------------------

;       CODE SEGMENT

;----------------------------------------------------------------------------

code segment

assume cs:code, ds:data

start:

mov ax, data

mov ds, ax

;--------------------------------------------------

; Display Header

;--------------------------------------------------

mov dx, offset msg1

mov ah, 09h

int 21h

;--------------------------------------------------

; BIOS Keyboard Status

; INT 16h Function 02h

; Returns keyboard flags in AL

;--------------------------------------------------

mov ah, 02h

int 16h

mov keystate, al

;--------------------------------------------------

; Caps Lock Status (Bit 6)

;--------------------------------------------------

mov dx, offset capsmsg

mov ah, 09h

int 21h

test keystate, 01000000b

jz caps_off

caps_on:

mov dx, offset onmsg

jmp caps_print

caps_off:

mov dx, offset offmsg

caps_print:

mov ah, 09h

int 21h

;--------------------------------------------------

; Num Lock Status (Bit 5)

;--------------------------------------------------

mov dx, offset nummsg

mov ah, 09h

int 21h

test keystate, 00100000b

jz num_off

num_on:

mov dx, offset onmsg

jmp num_print

num_off:

mov dx, offset offmsg

num_print:

mov ah, 09h

int 21h

;--------------------------------------------------

; Scroll Lock Status (Bit 4)

;--------------------------------------------------

mov dx, offset scrollmsg

mov ah, 09h

int 21h

test keystate, 00010000b

jz scroll_off

scroll_on:

mov dx, offset onmsg

jmp scroll_print

scroll_off:

mov dx, offset offmsg

scroll_print:

mov ah, 09h

int 21h

;--------------------------------------------------

; Insert Key Status (Bit 7)

;--------------------------------------------------

mov dx, offset insertmsg

mov ah, 09h

int 21h

test keystate, 10000000b

jz insert_off

insert_on:

mov dx, offset onmsg

jmp insert_print

insert_off:

mov dx, offset offmsg

insert_print:

mov ah, 09h

int 21h

;--------------------------------------------------

; Shift Key Status (Bits 0 and 1)

;--------------------------------------------------

mov dx, offset shiftmsg

mov ah, 09h

int 21h

test keystate, 00000011b

jz shift_off

shift_on:

mov dx, offset onmsg

jmp shift_print

shift_off:

mov dx, offset offmsg

shift_print:

mov ah, 09h

int 21h

;--------------------------------------------------

; Ctrl Key Status (Bit 2)

;--------------------------------------------------

mov dx, offset ctrlmsg

mov ah, 09h

int 21h

test keystate, 00000100b

jz ctrl_off

ctrl_on:

mov dx, offset onmsg

jmp ctrl_print

ctrl_off:

mov dx, offset offmsg

ctrl_print:

mov ah, 09h

int 21h

;--------------------------------------------------

; Alt Key Status (Bit 3)

;--------------------------------------------------

mov dx, offset altmsg

mov ah, 09h

int 21h

test keystate, 00001000b

jz alt_off

alt_on:

mov dx, offset onmsg

jmp alt_print

alt_off:

mov dx, offset offmsg

alt_print:

mov ah, 09h

int 21h

;--------------------------------------------------

; Exit Program

;--------------------------------------------------

mov ah, 4Ch

int 21h

code ends

end start

OUTPUT: -

AIM: -

To display the starting address of EBDA and Size of the EBDA

ALGORITHM: -

·  Initialize the data segment.

·  Display the header message.

·  Read the EBDA segment address from the BIOS Data Area at memory location 0040:000E.

·  Call the subroutine to convert and display the segment address in hexadecimal.

·  Calculate the 32-bit physical address by shifting the segment address left by 4 bits (multiplying by 16).

·  Call the subroutine to convert and display the physical address in hexadecimal.

·  Read the first byte of the EBDA segment to retrieve its size in Kilobytes.

·  Display the EBDA size in both hexadecimal and formatted string outputs.

·  Terminate program execution.

PROGRAM:-

TITLE   EBDA.ASM -- Display EBDA Start Address and Size

;//***********************************************************************

;//*                                                                     *

;//*   Copyright (c) 1985-2025, American Megatrends International LLC.   *

;//*                                                                     *

;//*      All rights reserved. Subject to AMI licensing agreement.       *

;//*                                                                     *

;//***********************************************************************

;<AMI_FHDR_START>

;

; Header:

; Author: Muthuganesh S

; Description:

; Program to display the starting address and size

; of Extended BIOS Data Area (EBDA).

;

;<AMI_FHDR_END>

.model small

.386

.stack 100h

;----------------------------------------------------------------------------

;       DATA SEGMENT

;----------------------------------------------------------------------------

data segment

msg1 db 13,10,"EBDA Information",13,10,13,10,'$'

msg2 db "EBDA Segment Address : 0x$"

msg3 db 13,10,"EBDA Physical Address: 0x$"

msg4 db 13,10,"EBDA Size            : 0x$"

hexbuf db '0000$'

hexbuf32 db '00000000$'

ebda_seg dw ?

ebda_size db ?

data ends

;----------------------------------------------------------------------------

;----------------------------------------------------------------------------

;       CODE SEGMENT

;----------------------------------------------------------------------------

code segment

assume cs:code, ds:data

start:

mov ax, data

mov ds, ax

;--------------------------------------------------

; Print Header

;--------------------------------------------------

mov dx, offset msg1

mov ah, 09h

int 21h

;--------------------------------------------------

; EBDA Segment Pointer

; BIOS Data Area

; 0040:000E

;--------------------------------------------------

mov ax, 0040h

mov es, ax

mov ax, es:[000Eh]

mov ebda_seg, ax

;--------------------------------------------------

; Print EBDA Segment Address

;--------------------------------------------------

mov dx, offset msg2

mov ah, 09h

int 21h

mov ax, ebda_seg

call PrintHexWord

;--------------------------------------------------

; Calculate Physical Address

; Physical = Segment * 16

;--------------------------------------------------

movzx eax, ebda_seg

shl eax, 4

;--------------------------------------------------

; Print Physical Address

;--------------------------------------------------

mov dx, offset msg3

mov ah, 09h

int 21h

call PrintHexDword

;--------------------------------------------------

; Read EBDA Size

; First byte of EBDA contains size in KB

;--------------------------------------------------

mov ax, ebda_seg

mov es, ax

mov al, es:[0000h]

mov ebda_size, al

;--------------------------------------------------

; Print EBDA Size

;--------------------------------------------------

mov dx, offset msg4

mov ah, 09h

int 21h

xor ah, ah

mov al, ebda_size

call PrintHexWord

mov dl, 'h'

mov ah, 02h

int 21h

mov dl, ' '

int 21h

mov dl, '('

int 21h

mov al, ebda_size

add al, '0'

mov dl, al

int 21h

mov dl, 'K'

int 21h

mov dl, 'B'

int 21h

mov dl, ')'

int 21h

;--------------------------------------------------

; Exit Program

;--------------------------------------------------

mov ah, 4Ch

int 21h

;----------------------------------------------------------------------------

; PrintHexWord Procedure

;----------------------------------------------------------------------------

PrintHexWord proc

push ax

push bx

push cx

push dx

push si

mov si, offset hexbuf

mov cx, 4

hex_loop1:

rol ax, 4

mov bl, al

and bl, 0Fh

cmp bl, 9

jbe digit1

add bl, 7

digit1:

add bl, '0'

mov [si], bl

inc si

loop hex_loop1

mov dx, offset hexbuf

mov ah, 09h

int 21h

pop si

pop dx

pop cx

pop bx

pop ax

ret

PrintHexWord endp

;----------------------------------------------------------------------------

; PrintHexDword Procedure

;----------------------------------------------------------------------------

PrintHexDword proc

push eax

push ebx

push ecx

push edx

push si

mov si, offset hexbuf32

mov cx, 8

hex_loop2:

rol eax, 4

mov bl, al

and bl, 0Fh

cmp bl, 9

jbe digit2

add bl, 7

digit2:

add bl, '0'

mov [si], bl

inc si

loop hex_loop2

mov dx, offset hexbuf32

mov ah, 09h

int 21h

pop si

pop edx

pop ecx

pop ebx

pop eax

ret

PrintHexDword endp

code ends

end start

OUTPUT: -

AIM: -

To write a program to verify the presence and no. of Option ROM’s available in the system and transfer the program control to VGA OpROM and regain the control back in the program.

ALGORITHM: -

·  Initialize the data segment.

·  Display the scanning start message.

·  Set the initial memory segment pointer to C000h.

·  Check the first two bytes of the current segment for the valid Option ROM signature (55h and AAh).

·  If a valid signature is found, increment the ROM counter and display the detected segment address.

·  If the detected ROM is at segment C000h (VGA ROM), execute a far call to C000:0003 and regain control afterward.

·  Read the ROM size byte at offset 2 and convert it into a segment increment value.

·  Add the increment value to the current segment pointer to jump to the next potential ROM boundary.

·  Repeat the scanning loop until the segment pointer reaches F000h.

·  Display the total count of discovered Option ROMs.

·  Terminate program execution.

PROGRAM:-

TITLE   OPROM.ASM -- Detect Option ROM and Transfer Control to VGA ROM

;//***********************************************************************

;//*                                                                     *

;//*   Copyright (c) 1985-2025, American Megatrends International LLC.   *

;//*                                                                     *

;//*      All rights reserved. Subject to AMI licensing agreement.       *

;//*                                                                     *

;//***********************************************************************

;<AMI_FHDR_START>

;

; Header:

; Author: Muthuganesh S

; Description:

; Program to scan Option ROM area, detect available

; Option ROMs, count them and transfer control to

; VGA Option ROM and regain control back.

;

;<AMI_FHDR_END>

.model small

.386

.stack 100h

;----------------------------------------------------------------------------

;       DATA SEGMENT

;----------------------------------------------------------------------------

data segment

msg1 db 13,10,"Scanning Option ROM Area...",13,10,'$'

msg2 db 13,10,"Option ROM Found at Segment : $"

msg3 db 13,10,"Total Option ROM Count      : $"

msg4 db 13,10,"Calling VGA Option ROM...",13,10,'$'

msg5 db 13,10,"Returned From VGA Option ROM",13,10,'$'

hexbuf db '0000$'

countbuf db '00$'

romcount db 0

data ends

;----------------------------------------------------------------------------

;----------------------------------------------------------------------------

;       CODE SEGMENT

;----------------------------------------------------------------------------

code segment

assume cs:code, ds:data

start:

mov ax,data

mov ds,ax

;--------------------------------------------------

; Print Start Message

;--------------------------------------------------

mov dx, offset msg1

mov ah,09h

int 21h

;--------------------------------------------------

; Start Scan from C000h

;--------------------------------------------------

mov ax,0C000h

mov es,ax

scan_loop:

;--------------------------------------------------

; Check ROM Signature 55AAh

;--------------------------------------------------

mov bx,0

mov al, es:[bx]

cmp al,55h

jne next_segment

mov al, es:[bx+1]

cmp al,0AAh

jne next_segment

;--------------------------------------------------

; Valid ROM Found

;--------------------------------------------------

inc romcount

mov dx, offset msg2

mov ah,09h

int 21h

mov ax, es

call PrintHexWord

;--------------------------------------------------

; VGA ROM Usually at C000h

;--------------------------------------------------

mov ax, es

cmp ax,0C000h

jne skip_vga

mov dx, offset msg4

mov ah,09h

int 21h

;--------------------------------------------------

; Transfer Control to VGA ROM

; Entry Point = C000:0003

;--------------------------------------------------

pushf

db 09Ah

dw 0003h

dw 0C000h

mov dx, offset msg5

mov ah,09h

int 21h

skip_vga:

next_segment:

;--------------------------------------------------

; Get ROM Size

; Offset 2 = Size in 512-byte blocks

;--------------------------------------------------

mov al, es:[0002h]

cmp al,0

jne valid_size

mov al,1

valid_size:

xor ah,ah

;--------------------------------------------------

; Convert to Segment Increment

; 512 bytes = 20h paragraphs

;--------------------------------------------------

mov bx,20h

mul bx

;--------------------------------------------------

; ES = ES + Size

;--------------------------------------------------

mov bx,es

add bx,ax

mov es,bx

;--------------------------------------------------

; Continue Until F000h

;--------------------------------------------------

mov bx,es

cmp bx,0F000h

jb scan_loop

;--------------------------------------------------

; Print Total ROM Count

;--------------------------------------------------

mov dx, offset msg3

mov ah,09h

int 21h

mov al, romcount

call PrintHexByte

;--------------------------------------------------

; Exit Program

;--------------------------------------------------

mov ah,4Ch

int 21h

;----------------------------------------------------------------------------

; PrintHexWord Procedure

;----------------------------------------------------------------------------

PrintHexWord proc

push ax

push bx

push cx

push dx

push si

mov si, offset hexbuf

mov cx,4

hex_loop1:

rol ax,4

mov bl,al

and bl,0Fh

cmp bl,9

jbe digit1

add bl,7

digit1:

add bl,'0'

mov [si],bl

inc si

loop hex_loop1

mov dx, offset hexbuf

mov ah,09h

int 21h

pop si

pop dx

pop cx

pop bx

pop ax

ret

PrintHexWord endp

;----------------------------------------------------------------------------

; PrintHexByte Procedure

;----------------------------------------------------------------------------

PrintHexByte proc

push ax

push bx

push dx

mov bl,al

shr al,4

call HexAscii

mov countbuf[0],al

mov al,bl

and al,0Fh

call HexAscii

mov countbuf[1],al

mov dx, offset countbuf

mov ah,09h

int 21h

pop dx

pop bx

pop ax

ret

PrintHexByte endp

;----------------------------------------------------------------------------

; HexAscii Procedure

;----------------------------------------------------------------------------

HexAscii proc

cmp al,9

jbe digit2

add al,7

digit2:

add al,'0'

ret

HexAscii endp

code ends

end start

OUTPUT: -