TITLE: -  KEYBOARD DISABLE

AIM: -

To write a program to disable Keyboard for 5 seconds by masking interrupts and enable it back

ALGORITHM: -

·  Initialize data segment.

·  Display keyboard disable message.

·  Read and save current PIC interrupt mask from port 21h.

·  Set bit 1 of the mask to disable IRQ1 (Keyboard) and write to port 21h.

·  Read current BIOS timer tick from memory address 0040:006C.

·  Add 91 ticks to the current timer value to set a 5-second target time.

·  Loop continuously until the memory timer tick reaches the target time.

·  Restore the original saved PIC mask to port 21h to re-enable the keyboard.

·  Display keyboard enable message.

·  Terminate program execution.

PROGRAM:-

TITLE   KEYBOARD.ASM -- Disable Keyboard for 5 Seconds Using PIC Masking

;//***********************************************************************

;//*                                                                     *

;//*   Copyright (c) 1985-2025, American Megatrends International LLC.   *

;//*                                                                     *

;//*      All rights reserved. Subject to AMI licensing agreement.       *

;//*                                                                     *

;//***********************************************************************

;<AMI_FHDR_START>

;

; Header:

; Author: Muthuganesh S

; Description:

; Disable keyboard interrupt (IRQ1) for 5 seconds by masking

; PIC interrupt and enable it back.

;

;<AMI_FHDR_END>

.model small

.386

.stack 100h

;----------------------------------------------------------------------------

;       DATA SEGMENT

;----------------------------------------------------------------------------

data segment

msg1 db 13,10,"Keyboard Disabled for 5 Seconds...",13,10,'$'

msg2 db 13,10,"Keyboard Enabled Back.",13,10,'$'

oldmask db ?

data ends

;----------------------------------------------------------------------------

;----------------------------------------------------------------------------

;       CODE SEGMENT

;----------------------------------------------------------------------------

code segment

assume cs:code, ds:data

start:

mov ax, data

mov ds, ax

;--------------------------------------------------

; Print Disable Message

;--------------------------------------------------

mov dx, offset msg1

mov ah, 09h

int 21h

;--------------------------------------------------

; Read Current PIC Mask

; PIC Master Mask Port = 21h

; IRQ1 = Keyboard Interrupt

;--------------------------------------------------

in al, 21h

mov oldmask, al

;--------------------------------------------------

; Mask Keyboard Interrupt (IRQ1)

; Set Bit1 = 1

;--------------------------------------------------

or al, 00000010b

out 21h, al

;--------------------------------------------------

; Delay for 5 Seconds

; BIOS Timer Tick:

; 18.2 ticks/sec

; 5 sec ≈ 91 ticks

; BIOS Data Area Timer = 0040:006C

;--------------------------------------------------

mov ax, 0040h

mov es, ax

mov eax, es:[006Ch]

add eax, 91

wait_loop:

cmp es:[006Ch], eax

jl wait_loop

;--------------------------------------------------

; Restore Original PIC Mask

;--------------------------------------------------

mov al, oldmask

out 21h, al

;--------------------------------------------------

; Print Enable Message

;--------------------------------------------------

mov dx, offset msg2

mov ah, 09h

int 21h

;--------------------------------------------------

; Exit Program

;--------------------------------------------------

mov ah, 4Ch

int 21h

code ends

end start

OUTPUT: -